CHENNAI EV CHARGING STATION DATA

EV-Agent – Agentic AI Powered Smart EV Charging Platform

Purpose: Reference dataset for the EV-Agent prototype, covering EV charging-station locations in Chennai, Tamil Nadu. Coordinate-level records are intended as seed data; live charger availability must be verified through current operator/TNEV sources before being represented as real-time.

# 1. Data Source and Usage Notes

Primary coordinate reference: Bureau of Energy Efficiency (BEE) public EV charging-station dataset.

Current-status reference: Tamil Nadu TNEV / TNGECL charger-finder platform and individual charging-network operator listings.

This document separates location information from live availability. A station being listed at a location does not mean a charger is available at this moment.

Approximate coordinates from secondary listings should not be treated as production-grade geospatial coordinates until independently verified.

For the EV-Agent prototype, store data_source, last_verified_at, and is_demo fields for every station.

# 2. Chennai EV Charging Station Reference Data

| No. | Operator | Station / Location | Area | Latitude | Longitude | Connector / Type | Power |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ather | St. Thomas Mount Railway Station | St. Thomas Mount | 12.994410 | 80.199288 | LEV DC | 3.3 kW |
| 2 | Ather | 10th Cross St, Indira Nagar | Adyar | 12.996663 | 80.251779 | LEV DC | 3.3 kW |
| 3 | Ather | Ather Space, Mount Road | Guindy | 13.012477 | 80.220603 | LEV DC | 3.3 kW |
| 4 | Ather | Kotturpuram Railway Station | Kotturpuram | 13.013870 | 80.248060 | LEV DC | 3.3 kW |
| 5 | Tata Power | Chennai One | Thoraipakkam | 12.948492 | 80.232697 | Type 2 AC | 7.4 kW |
| 6 | Tata Power | Chennai One | Thoraipakkam | 12.945845 | 80.231571 | Type 2 AC | 7.4 kW |
| 7 | Tata Power | Covelong Beach | Covelong | 12.797170 | 80.247583 | Type 2 AC | 7.4 kW |
| 8 | Tata Power | Tanishq / 100 Cathedral Road | Gopalapuram | Approx. | Approx. | CCS2 / AC | Varies |
| 9 | Tata Power | The Park / 601 Anna Salai | Nungambakkam | Approx. | Approx. | CCS2 | Varies |
| 10 | Tata Power | 164 Dr. Radhakrishnan Salai | Mylapore | Approx. | Approx. | CCS2 + Type 2 | 30 / 7 kW |
| 11 | Zeon | X Raptee HV HQ | Mugalivakkam | Approx. | Approx. | CCS2 | 30 kW |
| 12 | Zeon | RMZ One Paramount | Porur | Approx. | Approx. | CCS2 | 30 kW |
| 13 | Zeon | Pantheon Road | Egmore | Approx. | Approx. | CCS2 | 60 kW |
| 14 | Zeon | Darmapuram, 6th Street | T. Nagar | Approx. | Approx. | CCS2 | 30 kW |
| 15 | Zeon | Domestic Airport Parking C-03 | Meenambakkam | Approx. | Approx. | Type 2 | 7.4 kW |
| 16 | Zeon | Radisson Blu Hotel | Palavanthangal | Approx. | Approx. | Type 2 | 7.4 kW |
| 17 | Yocharge | Anna Salai | Central Chennai | Approx. | Approx. | CCS2 | 60 kW |
| 18 | Statiq | Express Estate | Royapettah | Approx. | Approx. | CCS2 | 60 kW |
| 19 | Mahindra | Egmore Metro Car Parking | Egmore | Approx. | Approx. | CCS2 | Varies |
| 20 | TEDA | NPKRR Maaligai, 144 Anna Salai | Thousand Lights | Approx. | Approx. | AC | Varies |

# 3. Recommended Database Fields for EV-Agent

| Field | Description |
| --- | --- |
| station_id | Unique station identifier |
| station_name | Charging-station name |
| operator | Charging-network/operator name |
| address | Full station address |
| city | Chennai |
| area | Locality / neighborhood |
| latitude | Station latitude |
| longitude | Station longitude |
| phone | Station/contact number, if available |
| opening_time | Opening time |
| closing_time | Closing time |
| is_24_hours | 24-hour operating flag |
| station_status | Overall station status |
| charger_id | Unique charger identifier |
| connector_type | CCS2, Type 2, LEV DC, etc. |
| charger_type | AC or DC |
| power_kw | Charging power in kW |
| status | Available, occupied, reserved, maintenance, offline |
| price_per_kwh | Price where verified |
| last_verified_at | Timestamp of most recent verification |
| data_source | BEE, TNEV, operator, etc. |
| is_demo | Whether the record is simulated/demo data |

# 4. Data Quality Classification

Use the following classification in the EV-Agent database:

VERIFIED_COORDINATE: Exact latitude/longitude obtained from a coordinate-bearing source.

OPERATOR_LISTING: Station reported by an operator or current directory, requiring coordinate verification if exact coordinates are not supplied.

LIVE_STATUS: Current availability/status obtained from a live or current charging-network source.

ESTIMATED: Value calculated by EV-Agent, such as estimated travel time or charging cost.

DEMO: Simulated data used for development/testing and not presented as live infrastructure.

# 5. Important Implementation Rule

Do not use approximate coordinates as final production coordinates. Before inserting a station into the production database, verify the exact location and current operational status through TNEV/TNGECL, the relevant charging operator, or another authoritative current source. Store the verification timestamp and source.

# 6. Suggested JSON Record Structure

{
  "station_id": "CHN-ZEON-PORUR-001",
  "station_name": "Zeon Charging Station Porur",
  "operator": "Zeon",
  "city": "Chennai",
  "area": "Porur",
  "latitude": 13.035,
  "longitude": 80.156,
  "connector_type": "CCS2",
  "charger_type": "DC",
  "power_kw": 30,
  "status": "AVAILABLE",
  "price_per_kwh": null,
  "is_demo": false,
  "data_source": "operator_listing",
  "last_verified_at": null
}

# 7. References

Bureau of Energy Efficiency (BEE) – EV Public Charging Station dataset.

Tamil Nadu TNEV / TNGECL – EV charger discovery and charging infrastructure platform.

Charging-network operator listings including Ather, Tata Power, Zeon, Statiq and Yocharge.

EV-Agent | Chennai EV Charging Reference Dataset